Elin Alcohol Caspian Co.

| درخواست کننده: شرکت محترم ---- | نام و نوع فرآورده: الکل اتیلیک طبی 96% |
| --- | --- |
| نوع بسته بندی: فله | نام تجاری: الین |
| 06/10/1404 تاریخ تهیه گزارش: | نام شرکت / واحد تولیدی: الین الکل کاسپین |
| تاریخ انقضا: 5 سال | تاریخ تولید: 06/10/1404 |
| تاریخ ارسال:--- | ETسری ساخت: 14041006 |
| IRC:4263903736218707 |  |

Analysis Table:

| Result | Specification | Monograph | Test |
| --- | --- | --- | --- |
| 0.814 | 0.812-0.816 @ 15.56 ˚c | Meets USP& conforms EP/BP38 | Apparent specific gravity |
| 0.806 | 0.805 - 0.812 @ 20 ˚c | Meets USP& conforms EP/BP38 | Relative density |
| 96.2 | Not less than 96 % V | Harmonized USP/BP38 | Assay (Ethanol) v/v 20 ˚c |
| Pass | It complies with the test for specific gravity It complies with the test for relative gravity | USP/BP38 | Identification test A |
| Pass | Conforms to IR Spectra | USP/BP38 | Identification test B |
| Pass | An intense blue color appears on the paper and becomes paler after 10 – 15 minutes | USP/BP38 | Identification test C |
| Pass | A yellow precipitate is formed within 30 minutes | USP/BP38 | Identification test D |
| Pass | The test solution has the appearance of water or is not more intensely colored than the standard solution | USP | Color of solution |
| Pass | The solution A&B show the same clarity than of water or their opalescence is not more pronounced than that of reference suspension A | USP/BP38 | Clarity of solution |
| Pass | Colorless/Clear/volatile liquid/mobile | USP | Description |
| Pass | Miscible with water and with practically all organic solvents | USP | Solubility |
| Pass | The solution is pink(30 ppm .expressed as acetic acid) | USP/EP/BP38 | Acidity or Alkalinity |
| <10ppm | Less than 2.5 mg of residue (25 ppm) | USP/EP/BP38 | Limit of nonvolatile residue |
| <10ppm | Less than 2.5 mg of residue (25 ppm) | USP/EP/BP38 | Residue on Evaporation |
| 0.05 0.08 0.06 0.05 0.05 Pass | 240 nm 0.40 max 250 nm 0.30 max 260 nm 0.30 max 270 nm 0.10 max 340 nm 0.10 max Absorption curve between 235 nm – 340 nm is smooth | USP/EP/BP38 | UV absorbance |
| ND <4 ppm ND <21ppm | Methanol (5mg/L max) Sum of Acetal and Acetaldehyde (10 mg/L max) Benzene ( 2 mg/L) Sum of all secondary peaks ( 300mg/L) | USP/EP/BP38 | Volatile impurities |
| According to <232> and <233> in USP or 2.4.20 in BP ND |  | USP/BP | Elemental Imp. |

*BP38: corresponding to British Pharmacopoeia/*USP46: corresponding to United State Pharmacopoeia

*This product meets or exceeds the current BP/USP specifications

GMP/ISO 9001-13485

QC manager\Technical Assistant   :                                                                   Plant manager: